REPORT RD0

RISK DEFINITION

TECHNICAL RISK ANALYSIS

| REFERENCE: 105698 |  |  | TIS AGENCY: CPV ARABIA |
| --- | --- | --- | --- |
| TECHNICAL/S RESPONSIBLE/S: DESIGN: Mohamed Aboelmakarem SITE INSPECTIONS: Ahmed Mansour (CIVIL ENGINEERING) |  |  |  |
| REVISION: 0 | CONTACT: Mahmoud Elmasry. |  |  |
| DATE OF ISSUE: 02-Sep-2024 | FAX: | PHONE: 00966553304612 | EMAIL: mmasry@cpvarabia.com |

TITLE I

PRINCIPAL/OWNER: ﻣﺆﺳﺴﺔ ﻣﻬﺎﺭﺗﻲ ﻟﻠﺘﻄﻮﻳﺮ ﺍﻟﻌﻘﺎﺭﻱ PROPOSER (IF DIFFERENT): ﻣﺆﺳﺴﺔ ﺧﺒﺮﺓ ﺍﻻﻧﺠﺎﺯ ﻟﻠﻤﻘﺎﻭﻻﺕ ﺍﻟﻌﺎﻣﺔ. PROJECT TITLE: 01 ﻣﻮﻗﻊ ADDRESS: ﺍﻟﻌﺎﺭﺽ. PROPOSED USE/OCCUPATION: المباني السكنية. NUMBER OF BUILDINGS: 2

SCOPE OF MISSION: DESIGN DESIGN + SITE INSPECTIONS - Date of TIS involvement: 02-Sep-2024 - Inspections from the commencement of Works YES NO - Missions: S W.1 E W.2 Q W.3 X, specify: S: Solidity and stability, including the envelope elements W.x: Waterproofing (1- Roofs, 2- Façades, 3-Basements) E: Existing structures Q: Works already started X: Other The TIS controls, inspections and checks marked with an X above are based on as analysis of the project, according to the requirements of the demander.

PARTIES INVOLVED (Full name + Address) DESIGN:

- ARCHITECT: )ﻣﺤﻤﺪ ﻋﺒﺪﺍﻟﻌﺰﻳﺰﺍﻟﻤﻨﻴﻊ ﻟﻼﺳﺘﺸﺎﺭﺍﺕ ﺍﻟﻬﻨﺪﺳﻴﺔ)ﻫﻨﺪﺳﺔ ﻣﺪﻧﻴﺔ - STRUCTURAL DESIGNER: )ﻣﺤﻤﺪ ﻋﺒﺪﺍﻟﻌﺰﻳﺰﺍﻟﻤﻨﻴﻊ ﻟﻼﺳﺘﺸﺎﺭﺍﺕ ﺍﻟﻬﻨﺪﺳﻴﺔ)ﻫﻨﺪﺳﺔ ﻣﺪﻧﻴﺔ - SOIL REPORT: مكتب ومختبرات ديار الهندسة لفحص التربة والدراسات الجيوتقنية CONSTRUCTION:

- MAIN CONTRACTOR: ﻣﺆﺳﺴﺔ ﺧﺒﺮﺓ ﺍﻻﻧﺠﺎﺯ ﻟﻠﻤﻘﺎﻭﻻﺕ ﺍﻟﻌﺎﻣﺔ - SUBCONTRACTORS: - PROJECT SUPERVISOR: )ﻣﺤﻤﺪ ﻋﺒﺪﺍﻟﻌﺰﻳﺰﺍﻟﻤﻨﻴﻊ ﻟﻼﺳﺘﺸﺎﺭﺍﺕ ﺍﻟﻬﻨﺪﺳﻴﺔ)ﻫﻨﺪﺳﺔ ﻣﺪﻧﻴﺔ - QUALITY TESTING FIRMS: - OTHER (SPECIFY): COMMENTS ON REFERENCES OF ARCHITECTS, ENGINEERING CONSULTANTS AND CONTRACTORS OF THE OPERATION, PURPOSE OF THE INSPECTION, IF ANY: NO REFERENCES AVAILABLE

DOCUMENTS USED IN THIS REPORT - STRUCTURAL DRAWINGS. - ARCHITECTURAL DRAWINGS. - BILLS OF QUANTITY. - SOIL INVESTIGATION REPORT. - PROPOSAL FORM FOR INHERENT DEFECT INSURANCE. Is there any other document or information missing which is needed for the completion of this report? YES NO If YES, please specify: - Construction schedule - Type of concrete used for foundationCast in situ columns, has not been mentioned in the structure plans. - Type of steel reinforcement used for foundation, Cast in situ columns, Cast in situ slabs, has not been mentioned in the structure plans. - Ground slope % - Quality assurance procedures - Method statement

CODE TABLE

| NATURE OF WORKS | SOIL RISKS | STRUCTURE TYPE | SPECIFICATIONS |
| --- | --- | --- | --- |
| CODE N°1: CONSTRUCTION TYPE A DETACHED OR SEMI-DETACHED HOUSES UP TO GF+2 AND B-1 B TERRACED HOUSES UP TO GF+3 AND B-1 C1 RESIDENTIAL BUILDINGS FROM GF+4 TO GF+14 AND UP TO B-2 C2 RESIDENTIAL BUILDINGS FROM GF+15 AND/OR FROM B-3 D1 OFFICES, HOTELS, SCHOOLS D2 HOSPITALS, CLINICS D3 COMMERCIAL BUILDINGS, RESTAURANTS, SHOPPING MALLS E1 OTHER PUBLIC BUILDINGS: THEATERS, RELIGIOUS BUILDINGS, STATIONS E2 STADES F1 COMMON INDUSTRIAL BUILDINGS, FACTORIES F2 LOGISTIC PLATFORMS G1 RESERVOIRS G2 SPECIAL INDUSTRIAL BUILDINGS, SMOKESTACKS, TANKS, RETAINING WALLS, ETC H BRIDGES, FOOTBRIDGES, JUNCTIONS AND OTHER WORKS | CODE N°2: SLOPE 0; 1; 2; 3; 4. <5%; 5 to 10%; 10% to 20%; 20 to 30%; >30%. CODE N°3: GROUND WATER AND AGRESSIVENESS 0; no water table. 3; works below water table. 4; works above water table. N; non-aggressive water or soil. Y; aggressive water or soil. CODE N°4: FOUNDATIONS A; footings. B; raft. final settlement; 0; 1; 2; 3; 4. <2 cm; 2 to 5 cm; >5 cm. C; piles or contiguous piles wall. D; friction piles. E; diaphragm wall. F; shafts. depth; 0; 1; 2; 3; 4. 0 to 3 m; 3 to 10 m; 10 to 25 m; 25 to 30 m; >30 m. Z; Other foundation type. CODE N°5: GROUND SPECIFIC RISKS O; no ground specific risks. P; underground quarries, sinkholes, karsts. Q; mining ground. R; Anchored earth retaining structure. (h > 3 meters). S; underpinning, basement construction. T; ground consolidation (grouting compaction; dynamic compaction/vibration). U; hazardous storage, concentrated surcharge loads, embankment. V; Compressive layer. W; founding on fill. X; other ground risk. Y; at least 2 ground specific risk. Z; new foundation type. | CODE N°6: STRUCTURE A VERTICAL STRUCTURES MASONRY B REINFORCED CONCRETE CAST IN-SITU C PRECAST REINFORCED CONCRETE - PREFABRICATED IN FACTORY RESIDENTIAL BUILDINGS FROM GF+4 TO GF+14 AND UP TO B-2 D PRECAST REINFORCED CONCRETE - ON SITE E PRESTRESSED CONCRETE EXCL. POST-TENSIONED CONCRETE F STEEL WORKS - SITE ASSEMBLY G PRE-ASSEMBLED STEEL WORKS H ON SITE WELDING I TIMBER STRUCTURE X TRADITIONAL COMPOSITE STRUCTURE Y WORKS ON EXISTING STRUCTURES - BASEMENT CONSTRUCTION - NEW FLOORS Z NEW STRUCTURAL SYSTEMS INCL. POST-TENSIONING | CODE N°7: FACADE A; concrete wall or masonry. B; sandwich or timber panels. C; curtain walls – steel frame glazing. D; curtain walls – glazing fixed with glue. E; curtain walls – glazing fixed bolts. F; breathable glazed curtain wall. G; cable curtain wall. Z; other type. CODE N°8: HEIGHT 0; 1; 2; 3; 4. Height - Ho; <25m; 25 to 35m; 35 to 60m; 60 to 100m; >100m. Maximum Headroom - Hp; <8m; 8 to 15m; 15 to 35 m; 35 to 50m; >50m. CODE N°9: DEPTH 1; 2; 3; 4. Depth; <5m; 5 to 10m; 10 to 15m; >15m. CODE N°10: SPANS 0; 1; 2; 3; 4. Spans; <15m; 15 to 20m; 20 to 30m; 30 to 40m; >40m. Spans (timber); <10m; >10m. CODE N°11: CANTILEVER 0; 1; 2; 3; 4. Cantilever; None; 0 to 2m; 2 to 5m; 5 to 10m; >10m. CODE N°12: SITE EXPOSED 0; 1. Distance to the sea; >5km; <5km. CODE N°13: WND SENSITIVITY 0; Concrete or masonry structure. 1; Steel, timber or composite structure. CODE N°14: WATERPROOFING OF ROOFS A1; pitched roof - tiles. A2; pitched roof - steel roof-deck. A3; pitched roof - sandwich panels. B1; flat roof-non practicable normal waterproofing. B2; flat roof-non practicable inverted waterproofing. B3; flat roof-practicable normal waterproofing. B4; flat roof-practicable inverted waterproofing. CODE N°15: WATERPROOFING OF BASEMENT A; External treatment (PVC, bitumen based, polyurethane membranes). B; Crystallization technology by concrete additive. C; Other. Z; No basement. |

PROJECT OVERALL VIEW

| CODE | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BUILD.1 | A B | 0 | 0 Y | A B 1 | W | B | A | 0 0 | 1 | 0 | 0 | 0 | 0 | B4 | A |
| Concerning the code table above, please complete one line per building/structure and explain each code below. PLEASE EXPLAIN IF SEVERAL CODES INDICATED IN THE SAME CASE. CODE 1: The project is Residential. It consists of 3 floors with annex and 1 basement CODE 2: No value has been indicated in the geotechnical report, so it is estimated that due to the geology of the area it is less than 5%. CODE 3: Ground water level was not detected in the boreholes. Aggressive due to water or soil ( Chloride in Soil= 0.6 % , Sulfate in Soil= 0.6 % ) and the actual concrete compressive strength is not mentioned CODE 4: The foundation type: Isolated Footings,Raft Foundation,and the settlement value:25 mm CODE 5: Founding on replacement soil. CODE 6: reinforced concrete cast in-situ, in columns, reinforced concrete cast in-situ, in foundation, reinforced concrete cast in-situ, in ground floor slab, reinforced concrete cast in-situ, in last floor slabs CODE 7: Concrete wall or masonry CODE 8: The total height is:21 m and the maximum headroom (Hp):5m CODE 9: Level of Foundation from street level: -1 m CODE 10: The maximum span is, approximately: m CODE 11: No Cantilevers have been founded in the floor slabs CODE 12: The distance to the sea > 5 km CODE 13: Concrete or masonry structure CODE 14: Details of roof waterproofing of the project flat roof-practicable inverted waterproofing, CODE 15: External treatment (PVC, bitumen based,polyurethane membranes) Summary description of the work, nature of the foundations and of the structure, specify the particularities (for example the presence of a ground water level and its position in relation to the last basement, the presence of basements or existing buildings along an excavation). The project is Residential. It consists of 3 floors with annex and 1 basement , Total Building height = 21 m The project consists of reinforced concrete beams and columns, founded on Isolated Footings,Raft Foundation,The floor slab of all slabs, including roof slab, are 32cm thick reinforced concrete hollow block slabs with beams, and 15cm thick reinforced concrete solid slabs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

DRAWINGS Please annex the following drawings, if available: LOCATION GENERAL DRAWING PLANS SECTIONS FACADES & ROOFS & TERRACES FOUNDATIONS OTHER, PLEASE SPECIFY: THESE DRAWINGS ARE ATTACHED IN ANNEX I. DRAWINGS.

PROVISIONAL TOTAL COST OF WORKS Are Architect and Engineering Consultants fees included in the cost of the works:; YES NO. - Total cost of the works: 568,100.00 SAR - Structural works: 250,000.00 SAR - Envelope: 30,000.00 SAR - Non-structural works: 60,000.00 SAR - Equipment, fixtures, and fittings: 25,000.00 SAR - External works: 120,000.00 SAR - Design and professional fees: 9,000.00 SAR - Taxes: 74,100.00 SAR Is there any special equipment (industrial machinery used in the manufacture of products or services) included in the cost other than those usually used in common constructions?; YES NO. If so, please specify: Nature: Cost: DATES AND CONSTRUCTION TIMES Expected duration of work (months): 6months Date of commencement of construction: 20-Jul-2022 Date of the first on-site visit of the Inspection Agency: 04-Aug-2022 Expected date of practical completion: 31-Jan-2023

TITLE II

SITE DESCRIPTION Is the building likely to be flooded (river, lake or sea, ground water level)? YES NO Maximal level of the ground water level to ground floor and to the last basement: - Ground water level was not detected in the boreholes - Ground floor level is +0.45 m. - Number of basement:1 - Level of the last basement:(-4m) Is there a system to prevent the effect of the water under-pressure? NA YES NO If YES, please specify: Is the building located in a seismic area? YES NO If YES, could you specify the level of protection? (Specify: statutory or contractual?): statutory Acceleration value: PGA=g . Indicate in % the ground slope: No value has been indicated in the geotechnical report, so it is estimated that due to the geology of the area it is less than 5%.. If >15%, was the landslide risk evaluated in the Geotechnical Report and/or the design? NA YES NO If NO, please make a reserve in conclusions. Is the building located in a cyclone area? YES NO Wind speed considered in the calculations (km/h): approximately km/h Is snow load applicable? YES NO If so, snow load value: Is the site located in a hostile environment (<5 km sea, soil, and underground water)? YES NO If YES, specify the nature and the protection provided: The nature: Ground water level was not detected in the boreholes. Aggressive due to water or soil ( Chloride in Soil= 0.6 % , Sulfate in Soil= 0.6 % ) and the actual concrete compressive strength is not mentioned The protection provided: - Use of (SRC) sulfate resisting cement type 5, Minimum Cement Content 350 kg/m3, the maximum water-cement ratio in the concrete mix is 0.50, and the minimum reinforcement concrete cover is 75 mm according to the geotechnical report. - Foundation elements should be coated below the ground surface with Bitumen.

GEOLOGY - TOPOGRAPHY – FOUNDATIONS Has a geotechnical engineer been involved in the design? YES NO Is there a geotechnical report? YES NO (If YES, please indicate the scope, number, and type of geotechnical tests and describe the layers including thickness). Number of boreholes: 5 - Borehole 1: BH-1, depth of: 2m - Borehole 2: BH-2, depth of: 15m - Borehole 3: BH-3, depth of: 10m - Borehole 4: BH-4, depth of: 1m - Borehole 5: BH-5, depth of: 9m Type of tests and trials: - RQD (Rock Quality Designation) - Chemical Analysis Description of the layers including thickness:

Are the adopted foundations founded on natural soil? YES NO Depth of excavation: -2 m from the street level. Depth of soil supporting foundations: -1 m from the street level. Nature of soil supporting foundations: well graded sand with silt, yellowish brown, medium dense, dry. All earthworks to be performed shall be compliant with the recommendations indicated in the geotechnical report. Please describe the foundations (level of foundation, bearing capacity, bearing pressure): - Foundation type: Isolated Footings,Raft Foundation, - Bearing Capacity: 2 kg/cm2. Are the conclusions of the Geotechnical Report relevant? YES NO If NOT, please explain the reason: Is the ultimate settlement indicated in the geotechnical report? YES NO Please indicate the value: 25 mm. Is the foundation system in line with the soil report? YES NO If NOT, please precise the differences and the reason: Has the RD1 report been filled in? YES NO If YES, please mark below the reason: Presence of back fill or compressible or expansible layers being used as foundations for the works Presence of piles, diaphragm walls or shafts superior to 3 m deep Presence of ground slopes exceeding 30% or cliffs sides Risk of ground sliding after soils excavation Are additional surveys needed? YES NO If YES, please precise: - Shear Test Parameters

TITLE III

The items below must be descripted briefly but detailed enough to include any relevant information. The used technologies must be specified where is possible: traditional construction, prefabricated structural elements, steel/concrete mixed structures, pre-tensioned (factory or site), welding (factory or site), etc It is a traditional system. Please indicate if innovative materials or technics are used which means (completion of RD2 report mandatory). No innovative materials or technics are used. VERTICAL STRUCTURES Full description of the adopted model. Are there non-vertical loads/transfer structures? YES NO VERTICAL ELEMENTS (COLUMNS, BEARING WALLS) Nature (reinforced or prestressed concrete, metal, timber...): - Cast is situ Columns: Fc= Not Mentioned Fy= Not Mentioned HORIZONTAL STRUCTURAL ELEMENTS FOUNDATIONS Nature (concrete, masonry, ..): Cast in situ. - Use of (SRC) sulfate resisting cement type 5, Minimum Cement Content 350 kg/m3, the maximum water-cement ratio in the concrete mix is 0.50, and the minimum reinforcement concrete cover is 75 mm according to the geotechnical report. - Foundation elements should be coated below the ground surface with Bitumen. Footings fc`: Not Mentioned Fy: Not Mentioned FLOOR SLABS Nature (reinforced or prestressed concrete, metal, timber...): - Cast is situ slabs: Fc= Not Mentioned Fy= Not Mentioned - Maximum span for beams (m): 6 m - Maximum span for slabs (m): 5 m - Maximum cantilever of the balconies (m):2 m ${B307} ${B308} BALCONIES ${B312} ROOF STRUCTURAL ELEMENTS Please note if those are: FLAT PITCHED If flat, similar to floor slab? YES NO If NO indicate modifications: If pitched, please note its main characteristics:

HEAVY FACADES Type, total thickness, rendering, cladding... (Specify load-bearing facades or not, precast, or not.) Façade details: Concrete wall or masonry LIGHT-WEIGHT FACADES Not Applicable Frame (aluminum, steel, timber or other): Filling (glass, composite wall, in this case, specify the constitution): Total surface of each type of façade: Possibility to replace the façade elements easily: YES NO HORIZONTAL STABILITY Traditional structural elements (frame, inner walls, bracings...)? YES NO If not: fill in the complementary report RD2 Traditional structural elements are reinforced concrete frames. BASEMENT WATERPROOFING Presence of water (water flow, water table)? YES NO If YES: solution chosen (waterproofness, drainage, and shaft lining...) waterproofing system: FACADE WATERPROOFING

ROOF WATERPROOFING Roof details:

GROUND SLAB Is the ground slab suspended? YES NO If NOT, is the ground slab bearing on backfill? YES NO Maximum thickness of backfill: Loads: Spread: YES NO Value: not informed Concentrating/rolling: YES NO Value: Other: YES NO Value:

TITLE IV

IDENTIFIED AGGRAVATING RISKS AND ADDITIONAL INFORMATION: (For example, large span beams or slabs, foundation for heavy and/or vibrating machinery, etc.) It is necessary to submit the documents indicated in title I, referring to the documents used in this report. Design (RD5): - aaaaa bbbb - beam ad Inspection (RD5): - The length of the building is greater than 45 m. Technical reservations: - Not exist Notes: - level of water - The existence of a basement floor wall from bricks. - The existence of raft and another footing type working together in the same building was detected, it is probable that the raft will have a higher settlement than the footings, It is necessary to analyze in greater detail the differential settlements between both types of foundations. Aggravating risks: - It was not specified how the connection between the masonry of the facade and the concrete structure is made. - No specification has been found regarding insulation and waterproofing of the joints between different types of facades. - Type of steel reinforcement used for foundation, Cast in situ columns, Cast in situ slabs, has not been mentioned in the structure plans. - There are no details about lintels (dimension and reinforcement). - The soil study doesn't present shear test parameters. - In waterproofing roof system, the heat insulation can not be executed directly after membrane layer.

TECHNICAL STANDARDS USED IN THE PROJECT: - 301 Structural – Loading and Forces. - 302 Structural – Testing and Inspection. - 303 Structural – Soil and Foundations. - 304 Structural – Concrete Structures. - 305 Structural – Masonry Structures. - 306 Structural – Steel Structures. Are they relevant? YES NO If NOT, please specify why:

TITLE V

CONCLUSIONS

Additional reports needed to estimate the risks and issuing date: RD1 Special Foundation Report – Expected date: RD2 Innovative materials/procedures report – Expected date: RD3 Waterproofing assessment report (Waterproofing Certificate Approval) – Expected date: 31-Jan-2024 RD5 Works interruption report – Expected date: It will be issued to highlight major deviation noted on site during inspection or failure to submit the required documents RD6 Final risk assessment report (Certificate of Approval) – Expected date: 01-Feb-2023 RISK TECHNICAL ASSESSMENT (Risk assessment based on the checked documents. Please specify topics requiring special focus). Within the scope of its mission, the CPV, based on the technical documentation received from the project and after its general preliminary analysis and reflected in this report, it is considered that the TECHNICAL RISK IS AGGRAVATED However, If the questions raised in this report are clarified, some of the aggravating situations can be reduced to a normal risk situation. A list of AGGRAVATING RISKS is included in title IV of this document. technical reserves were opened:

Does the risk require the involvement of any TIS headquarter expert? YES NO If YES, specify the reason: Minimum number of site visits: 5 Of which: -Foundations visits: 01 -Structure visits: 02 -Waterproofing (if relevant): 01 -Final inspection: 01

Riyadh, 02-Sep-2024

THE TECHNICIAN/S IN CHARGE OF THE CONTROL

ENG. Mohamed Aboelmakarem

Civil engineer

Project Analysis Department Manager

ENG. MAHMOUD ELMASRY

Civil engineer

Quality Control Manager

ENG. Alaa Abdulkareem

Civil engineer

# ANNEX I. DRAWINGS.

LOCATION